Date of issue:

Issued by:

Sports Clip System Screeded Sub-floor      4 pages

NBS SPECIFICATION K21 CLAUSE 111

Project:				•

TIMBER FLOORING FLOATING/CLIP SYSTEM

Location:				•

Substrate & Preparation.             • New or (Delete as necessary) Existing screed base

to BS 8204 Part 1 2003, to have flatness tolerance

SR1, which should not deviate more than a 3mm gap

showing under a 2m straight edge having a

suitable damp proof membrane underneath.

• If block and beam suspended ground floor construction, ensure that the void is correctly ventilated to current Building Regulations with any earth base at least 225mm below any pre cast beams.

• As clause 260.

Resilient Layer/Underlay:	•Junckers 10mm thick Sports Foam underlay installed over membrane on top of the cementitious base. Conforming to BSEN14904:2006, category A3.

• Load bearing capacity 5.0kN/m2. For information on retractable seating loadings contact Junckers.

• As  clause 310

Moisture membrane:	• Junckers SylvaThene moisture barrier installed on top of the cementitious base.

• As clause 310

Boards:	• Junckers Solid Pre-finished hardwood, free from decay and insect attack.  Planed all round.

Comprising two rows of staves with unique double dovetail construction.

• Polyback, polythene or lacquer moisture balancer fitted to the underside.

Manufacturer                                • Junckers Ltd

Tel No 0044 1376 534737

E mail sales@junckers.co.uk

Web www.junckers.co.uk

Area manager contact – ........................

Tel No .............................

Wood species:		• Junckers Solid Press Dried Beech

Appearance class/Grade:	•. SylvaSport Champion, Premium or

Club or (Delete as necessary) SylvaTech plus

Champion, Premium or (Delete as  necessary) Club

Finished Face Width:		•   129mm.

Finished Thickness:		•      22mm.

Finished Length:		•   3700mm.

Edges:	• Tongue and grooved.   Square edge and wax edged sealed.

Moisture Content		• 7.5% + manufacturing tolerance.

at Time of Fixing:

Method of fixing/Joining:	• By Junckers proprietary Quick Clip System

( 17N°. clips per m²).  Floor to be laid in accordance with manufacturer’s recommendations.

(Clip size………….mm).

•  As clause 350.

Finish:	•  Factory finished with several coats of UV light cured lacquer and one coat of Junckers two-component polyurethane Isolacquer Silk Matt Lacquer (50% gloss), total film thickness 45 microns.

Accessories:	• Junckers 38mm x 48mm Combi Skirting.

• Junckers Bushings at post socket locations.

Other Requirements:	  • Court line marking to be carried out using Junckers Sport Linemarking paint.

• On completion apply one coat of Junckers HP800 Sport lacquer.

.

210		 WORKMANSHIP GENERALLY:

• Keep boards dry before installation.

• Methods of fixing and fasteners to be as section Z20 unless specified otherwise.

• Protect from dirt, stains and damage until Practical Completion using suitable coverings and boards laid as the work proceeds.

220					ENVIRONMENTALCONDITIONS:

• Do not start work specified in this section before building is weather tight, wet trades have finished their work and the building is well dried out.

• Before, during and after laying, temperature and humidity must be maintained at levels approximating to those which will prevail after building is occupied.

• Relative humidity must be maintained within the range of 35-65%RH. This must be provided prior to installation and maintained from there on.

Exchange of Information

Ensure that there is a system in place for proper exchange of information between the design team and all specialist contractors, including persons responsible for commissioning and hand over, to ensure that the correct environmental conditions are achieved and maintained.

- HEATING SYSTEM:

• Agree arrangements for operating the heating installation up to the date of Practical Completion of the Works to ensure that excessive moisture movement of flooring does not take place.

• The heating system has been designed to provide ………………. heating giving temperatures in the range …………...°C  to……………ºC.

- AIR CONDITIONING:

• Agree arrangements for operating the air conditioning installation up to the date of Practical Completion of the Works to ensure that excessive moisture movement of flooring does not take place.

• The system has been designed to provide ……………….……………………………………………..

- FIXTURES:

Do not start work specified in this section until any fixtures around which flooring is to be fixed have been installed.

- DRYNESS OF CONCRETE/SCREED SUBSTRATES FOR FLOORING:

• Where flooring is to be laid on a new or existing concrete or screed base, ensure that drying aids have been turned off for not less than 4 days, then

• On all concrete or screed bases test for moisture content, using an accurately calibrated hygrometer in accordance with BS8201, Appendix A.

•   Take readings in all corners, along edges, and at various points over the area being tested.

•   Do not lay flooring until all readings show 75% relative humidity or less.

- BOARD MOISTURE CONTENT TESTING:

When instructed by CA, test timber with an approved moisture meter to manufacturer’s recommendations. Provide CA with records all tests and results.

- CONTROL SAMPLE:

Complete sample area, being part of the finished work, in approved location as follows and obtain approval of appearance before proceeding:

Clause reference:……………………..

Location:………………………………

Size:……………………………………

Include features:………………………

- VAPOUR CHECK/MOISTURE MEMBRANE:

• To be lapped and taped, and turned up at perimeter walls.

• Ensure membrane is clean, dry and free from punctures and tears before laying flooring.

- FIXING BOARDS:

• Fix each board securely to give a flat true surface free from undulations, hammer marks, scratches and protruding fastenings.

• Allow for movement of timber through expansion or contraction by installing to manufacturer’s recommendations to suit the expected relative humidity range of building when in use.

• Header joints to be tongue and grooved, tightly butted and glued using Junckers SylvaFix header joint adhesive and set out to achieve a random effect.

- EXPANSION PROVISION:

• Expansion gaps:

• Edges of flooring: Parallel to lie of boards form a clear perimeter gap equivalent to 2.0mm per running metre width of flooring on either side and around pipes, columns etc. minimum size 15mm and to be covered by a Junckers 38mm x 48mm Combi skirting .

• Ends of flooring: A gap filled with Junckers 12mm x 12mm rubber strip, equivalent to 1mm per running metre length of flooring to be left at either end of the flooring.

• Remove any temporary spacer blocks and debris before fixing skirting/cover fillets.